Table of Contents

# ORANSlice Project Handoff Summary

Purpose of this document: a self-contained record of what this testbed set out to do, what was actually built and verified, what was learned (including the hard-won negative results), and what to carry over to a fresh Ubuntu + OAI build. Written to be read without the surrounding chat history.

Date: 2026-07-14 Repo state at handoff: commit c01c7b2 on feat/qoe-oran-framework-stage-zero-20260711-push (local; not yet pushed — no git credentials in this sandbox).

## 1. Project objective

The end goal is an empirical realization (paper #4) of a QoE-aware, GNN-MARL-FL O-RAN network-slicing framework (paper #3, an IEEE Access survey), grounded by two earlier conference papers:

- Paper #1 (MECON): DRL-based slice admission control (DQN / A2C / Rainbow), single-gNB, 2 slices (URLLC, eMBB).

- Paper #2 (ICRAIE): Extends #1 with a 3rd slice (mMTC) and a multi-gNB load-balancing term.

- Paper #3: The survey this whole line of work targets — QoE-aware admission via a learned QoS→MOS mapper, GNN-based state representation, multi-agent RL, federated learning across gNBs.

Given real hardware constraints (one physical rig, one gNB), the work was staged:

- Stage Zero — reproduce papers #1/#2’s admission-control setup against a real OAI testbed (not a simulator), establish the live E2 control loop, and calibrate a working baseline.

- Stage One — layer a QoE mapper on top of the frozen Stage Zero baseline (paper #3’s first real component), calibrated against real quality-of-experience standards, without touching anything else (“change one thing”).

- Stage Two+ (not started) — GNN state representation, multi-agent coordination, federated learning across gNBs — deferred until multiple physical gNBs exist.

## 2. Stage Zero: what was built and achieved

### What exists that didn’t before

A complete qoe_oran_framework/ package implementing the papers’ admission-control MDP against any KPM source (replayed trace, synthetic, or live), including: - env.py — gym-like RANEnv (reset()/step()) - kpm_adapter.py — real E2SM-KPM UE samples → per-slice aggregate state - reward.py — eq. 2 (paper #1) and its LB-extended form (paper #2) - action_mapping.py — binary accept/reject → the real control primitive (see §4 below) - live_kpm_source.py — a from-scratch, reverse-engineered UDP client for OAI’s real E2_AGENT (see §4) - policies/ — DQN, A2C, Rainbow admission policies (reusing base classes from drl_slicing/oranslice_drl) - comparators/lb_only_baseline.py — the papers’ own “lowest-utilization-gNB” heuristic baseline - omega_logger.py — every run logged as an {role, method, objective, constraint, evidence, limitation} tuple; every record’s limitation field is non-empty by construction - mc_runner.py — Monte-Carlo orchestration with fixed seeds, offline training and live evaluation modes - Full offline→live pipeline: scripts/train_offline.py, scripts/run_live_mc.py, xapp/saclb_xapp.py, scripts/run_saclb_live_testbed.sh

### Key findings (real, verified against the physical rig)

- First genuine live differentiation between policies was achieved (LB-only baseline vs. learned policies showing different block rates on real traffic).

- SLA-compliance differentiation between DQN and A2C proved structurally hard to achieve given this rig’s Lmax/offered-demand-volatility interaction — documented as an open limitation, not silently dropped.

- Full closeout report: qoe_oran_framework/STAGE_ZERO_REPORT.md / .docx (included in this handoff).

## 3. Stage One: what was built and achieved

### The QoE mapper (paper #3’s first real component)

- qoe_mapper.py: iqx_mos() (eq. 3, the IQX closed-form QoS→MOS curve), a stateful LatencyProxy (holds the last real reading + staleness count — see §4), RollingKpmWindow, and QoEMapper (an LSTM that predicts a bounded residual correction on top of the IQX prior, not a from-scratch regression).

- calibration/: per-slice objective-label generators — a P.1203 reference-implementation bridge (via a small virtual ABR video client) for eMBB, and ACR-style task-success scoring for URLLC/mMTC (P.1203 doesn’t apply to non-perceptual traffic) — plus fit_iqx.py (least-squares IQX coefficient fitting) and train_lstm.py (LSTM training on synthetic KPM-trajectory windows).

- Calibration results (real, held-out, reported honestly):

| - Slice | - IQX prior MAE / r | - LSTM-refined MAE / r |
| --- | --- | --- |
| - eMBB | - 0.089 / 0.926 | - 0.013 / 0.997 |
| - URLLC | - 0.131 / 0.742 | - 0.055 / 0.911 |
| - mMTC | - 0.347 / 0.961 | - 0.047 / 0.999 |

- URLLC’s weaker IQX-alone fit is a genuine, documented finding (its near-binary deadline profile doesn’t fit IQX’s smooth log curve as well as the other two) — the LSTM refinement closes most of that gap.

- reward_mode="qoe": eq. 9 (r = α·MOS − β·cost − γ·SLA_viol) threaded through env.py/mc_runner.py/all three entrypoints via a --reward-mode flag, a genuinely different reward shape from Stage Zero’s eq. 2, not eq. 2 with one term swapped. reward_mode="sla" (the frozen baseline) is regression-tested to be bit-for-bit unaffected even when the new QoE-diagnostic machinery is present in a config.

- DQN and A2C retrained offline under the QoE reward, 300 episodes × 2 seeds each, results reproducible (max ~1.5% variance between seeds).

### Live pre-flight evidence (real testbed, not simulated)

- Fig. 5 (the headline result): the actual PRB ratio ceiling (slicing_control_m max_ratio) each policy commands, logged every step. lb_only never leaves the configured floor for any slice, the entire episode; the QoE-trained DQN rides every slice’s ceiling to its cap within seconds and holds it there. This is the real controllable output, not an inferred effect.

- Fig. 6 (the honest caveat): that ceiling divergence has not yet propagated to backlog or inferred MOS at this rig’s traffic scale — both policies show nearly identical downstream QoE. Reported as a genuine limitation, not hidden.

- Two root-cause investigations, both concluded with mechanism, not guesswork:

  - Traffic-rate intensification (~15–30× heavier packet rate) does not change admission counts (driven by a seeded synthetic arrival stream, decoupled from real traffic by construction) or scheduler PRB grants (avg_prbs_dl saturates at the ceiling regardless of offered load) — ruled out.

  - UE-count scaling (more simultaneously-attached real UEs, to create genuine multi-flow contention) hit a confirmed host-memory ceiling on this shared desktop rig (~1GB RSS per simulated UE, verified via /proc/<pid>/status, tied to OAI’s real-time PHY buffer allocation at the configured bandwidth — not a tunable inefficiency) — ruled out on this machine, though it remains a valid lever on more capable hardware.

- A real bug was caught and fixed before it could waste live-rig time: MOS/cost/SLA-violation diagnostics were only computed under reward_mode="qoe", so the baseline arm’s live runs would have logged empty QoE data — fixed as a passive, reward-independent diagnostic, regression-tested to leave the baseline’s actual reward unchanged.

### Why Stage One’s live phase stopped here

A genuine OAI binary segfault started reproducing 100% of the time (even a single UE alone crashes within seconds of attach). Root-caused via dmesg + addr2line (correcting for the ELF’s PIE load-segment offset) to two exact locations — nr_dci_size and get_ul_tdalist in openair2/LAYER2/NR_MAC_COMMON/nr_mac_common.c — both reading a UE bandwidth-part config struct that’s NULL or incomplete. Ruled out multi-UE races, stale AMF state (survived a full AMF restart + re-provisioning), and port/process conflicts before concluding this is a genuine logic bug in the vendored OAI build, not an environment issue. See §5 for why this motivates the fresh-build decision.

## 4. Key technical discoveries (carry these forward — they cost real time to learn)

- Real OAI’s only xApp control primitive is slicing_control_m{sst, sd, min_ratio, max_ratio} — a per-slice PRB-ratio ceiling/floor. There is no raw “accept/reject this one flow” hook. The papers’ binary admission action has to be realized as ceiling nudging (accept → ceiling up, reject → ceiling down), not a literal per-flow accept. Capping a slice starves it (demand → backlog); it does not remove demand. This is the single most important architectural fact about this testbed, and will be true on any OAI version.

- OAI’s E2 agent has a RIC-free UDP loop, but it is easy to reach for the wrong transport. The correct, verified wire protocol (live_kpm_source.py’s docstring has the full detail): gNB listens on UDP 0.0.0.0:6655 for INDICATION_REQUEST/CONTROL protobufs, responds to UDP 127.0.0.1:6600. Request/response, not subscribe-and-stream. Sending a SUBSCRIPTION-type message crashes the gNB (assert(0!=0) in handle_subscription()) — never send one. The o-ran-e2sim/kpm_sim pipeline and xapp-oai/base-xapp’s TCP/UDP transport were both tried first and found to require a real E2AP/SCTP RIC Subscription Request before relaying data — i.e. they need an actual RIC, which this rig doesn’t have. live_kpm_source.py reimplements the socket calls directly and needs no external xApp framework. This wire protocol must be re-verified on the new OAI version — if the E2 agent’s message format changed, this file needs updating, but the discovery process (check e2_agent_app.c/e2_message_handlers.c directly, don’t trust example scripts) is the reusable part.

- There is no literal E2SM-KPM latency field. The closest real signal is dl_mac_buffer_occupation, wired to the real MAC scheduler’s backlog counter — genuine but intermittent (in one 3000-step live campaign: 99.9% of steps had a nonzero reading for eMBB, 53.3% for URLLC, 13.5% for mMTC). Treat it with a held/staleness-tagged proxy (qoe_mapper.py’s LatencyProxy), not a naive “0 this step = confirmed zero.”

- Every “will the ceiling ever bind” question needs a real-demand baseline first. Early config iterations left 10–20× headroom above real observed per-UE demand (~5 PRB/UE on this rig, avg_prbs_dl polled directly) — every policy just accepted everything, zero differentiation, for a long time before this was caught. Always poll real demand before setting ratio caps.

- Synthetic admission “requests” and real UE traffic are two decoupled layers. accepted_counts/block counts come from a seeded synthetic arrival stream (_synthesize_requests); real UE traffic only affects what the E2SM-KPM layer reports (PRB usage, backlog). Pushing more real packets can never change admission counts directly — only the observed state that a policy conditions its decisions on.

- A software UE process (nr-uesoftmodem, --rfsim) costs ~1GB resident RAM at 106 PRB/numerology 1, allocated at startup for real-time PHY buffers (FFT tables, HARQ soft-bit buffers) — confirmed via /proc/<pid>/status, not a leak or a tunable log-buffer setting. Budget hardware accordingly if genuine multi-UE contention testing matters to Stage One/Two — this rig (7.4GB RAM, shared with a live desktop session) tops out around 3–4 simulated UEs before swap thrashing degrades timing enough to risk exactly the kind of instability in §3.

## 5. Why a fresh build, not an in-place fix

The vendored OAI checkout is v2.1.0 (February 2024) — confirmed via oai_ran/CHANGELOG.md and the UE Docker image’s baked-in commit (c599e172, 2024-02-16) — notably the exact release that introduced the E2 agent this whole pipeline depends on. A web search of upstream’s commit history on the crashing file (nr_mac_common.c) found multiple segfault-prevention fixes dated through late 2025, suggesting real upstream progress on exactly this class of bug — but confirming a specific fix applies, and that a newer checkout’s E2 agent is still wire-compatible with live_kpm_source.py, would require either a large, uncertain-payoff cherry-pick or a full migration. Given that, doing a clean fresh build (latest Ubuntu LTS + latest stable OAI with E2 agent support) and re-verifying the E2 wire protocol from scratch is the more tractable path than patching a 2+-year-old checkout blind.

When standing up the new build, re-run the Stage Zero precondition checks before writing any new application code: (1) confirm E2 agent is present and reachable (check for E2_AGENT/e2_agent_app.c in the source tree, not just the CMake option name — some builds gate it differently); (2) re-verify the wire protocol against the new version’s e2_message_handlers.c directly; (3) re-poll real per-UE demand before setting any ratio caps; (4) re-characterize which KPM fields are actually populated live (§4.3) before wiring the QoE mapper’s inputs.

## 6. What’s in the migration bundle (oranslice_migration_bundle.zip)

Fully reusable as-is (pure Python, RAN-version-independent): - qoe_oran_framework/ — the complete framework: env.py, config.py, reward.py, qoe_mapper.py, action_mapping.py, kpm_adapter.py, mc_runner.py, omega_logger.py, replay_kpm_source.py, types.py, calibration/, policies/, comparators/, scripts/, tests/ (140 tests), configs/ (as reference — ratio caps/reward weights will need re-tuning against the new rig’s real demand, per §4.4) - drl_slicing/oranslice_drl/ — base RL classes (RLPolicy, QNetwork, ActorCriticNetwork, ReplayBuffer) that qoe_oran_framework’s policies import - drl_slicing/scripts/ — UE fleet generation (generate_ue_fleet_compose.py, including this session’s fixes for N-per-slice and custom contention profiles), subscriber provisioning, traffic-profile orchestration — the patterns are reusable even where OAI-version-specific flags need updating - qoe_oran_framework/results/qoe_mapper/ — trained LSTM checkpoints + fitted_iqx_coeffs.json (calibrated against P.1203/ACR standards, not this specific hardware — a reasonable warm-start prior, should be re-validated once live KPM behavior is re-characterized on the new build) - Small offline-trained DQN/A2C checkpoints (checkpoint.pt + summary.json only, not the large raw omega_log.jsonl traces — those are historical record, already in git, not meant for redeployment) — useful as an architecture/hyperparameter reference, will need retraining against the new rig’s actual dynamics

Needs re-verification, not blind reuse: - live_kpm_source.py — wire protocol may differ on a newer OAI E2 agent (§4.2) - All configs/*.yaml ratio caps / Lmax / reward weights — calibrated against this rig’s specific real demand (§4.4)

Not included (intentionally left behind — this is exactly what’s being replaced): oai_ran/, oai_cn/, docker_open5gs/, protobuf-c/, all live omega_log.jsonl traces, gNB/UE .conf files.

Documents included: this file, qoe_oran_framework/STAGE_ZERO_REPORT.md/.docx.